Demo: Deploy and Classify Webcam Images on NVIDIA Jetson Platform from Simulink

Prerequisites

Target Board Requirements

- NVIDIA Jetson embedded platform.

- Ethernet crossover cable to connect the target board and host PC. (if you cannot connect the target board to a local network)

- USB webcam connected to the USB host port of the target.

- A monitor connected to the display port of the target.

- V4L2 and SDL (v1.2) libraries on the target.

- GStreamer libraries on the target.

- NVIDIA CUDA® toolkit and driver.

- NVIDIA cuDNN library on the target.

- Environment variables on the target for the compilers and libraries. For more information, see Install and Setup Prerequisites for NVIDIA Boards.

  - For Install and Setup, choose correct software version based on your boards. If Jetson Nano use JetPack 4.6.1-  JetPack 4.6.1 Installer

  - Choose installing JetPack using SD Card Image Method. Download the SD Card Image and follow the steps at Get Started With Jetson Nano Developer Kit

  - Next continue to finish all setup as instructed on Install and Setup Prerequisites for NVIDIA Boards

  - Try to ping your board with your host computer.

    - Jetson address should be ###.###.###.2

    - Host address should be ###.###.###.3

    - Use direct ethernet, wifi may have firewalls

    - Disable all relevant firewalls and open port 22 for both outbound and inbound

  - If you receive error on SHH, try to consider to

    - Install openssh-client and openssh-server

      - $apt-get install openssh-client

      - $apt-get install openssh-server

      - $systemctl ssh start

      - $systemctl ssh status

    - Possible you need to regenerate ssh host keys

      - Delete ssh keys by $sudo/bin/rm-v/etc/ssh/ssh_host_*

      - Regenarate by $sudo dpkg-reconfigure openssh-server

      - Then delete ssh keys in host

      - Ssh to jetson to check connection by passing >>ssh<username>@<host_ip>

Development Host Requirements

- Environment variables for the compilers and libraries. For more information, see Third-Party Hardware (GPU Coder) and Setting Up the Prerequisite Products (GPU Coder).

Connect to NVIDIA Jetson

- Make sure no error/warning at this point.

- Verify the GPU environment. You should not received any error

- Identify camera connected to target board

- Set the camName and camResolution.

- Open Simulink model

- Open camera block and make sure camera name and resolution is same as in MATLAB code

- Open hardware setting. Make sure all setting is correct

- Open build options and change the build directory to /home/ubuntu. Before that make sure you have a directory in your Jetson. How to create is by

  - $cd/home

  - $mkdir ubuntu

  - $chmod ugo + rwx /home/ubuntu

  - >>exit

- Change also the display. Please check the display that available on your jetson. How to check is by

  - $echo $DISPLAY

  - If it display 1 change the display to 1, if it 0 change the display to 0

- Next step, open interface and change the deep learning target library to TensorRT instead of cuDNN. TensorRT will give a good frame per second compared to cuDNN.

- Click Apply

- Click Build, Deploy & Start

- Open home/ubuntu file in Jetson Nano, you will notice that .elf file has been created.

- Run that file on terminal